FS4000,PTOBC288, and BC283
Brush cutter rotor comparisons

| QUALITIES: | FIXED TOOTH | FLAIL CUTTER |
| --- | --- | --- |
| MULCH | 90 % shards, ¾” dia. X 5” Long | 90% ¾” chips |
| SPEED | 85% of flail | Fastest |
| TOOTH LIFE | 300-400 hours (at best) | 100-200 hours (at best) |
| SHARPENING NEEDED | No | Yes, at about half of life |
| SURFACE ROCK | Yes | No |
| CUTS BELOW GROUND LEVEL | Yes | No |
| BELT LIFE | 75% of flail | Longest |
| BEARING LIFE | 75% of flail | Longest |
| SHAFT LIFE | 75% of flail | Longest |
| WEIGHT | Highest | Lowest |

FIXED TOOTH PHOTOS:

Rotor Head with Fixed Teeth 					Close-up on single tooth

FLAIL CUTTER PHOTOS:

Rotor Head with Flail Cutters					Close-up on single flail cutter